INVITATION TO BID (ITB)

Ref: ITB-TEF-2026-002

Project: TRIPOLI EMERGENCY FUND – Consolidation and Structural Rehabilitation of Building (Plot No. 2054 – Basatin Trablous)

Issue Date: Tuesday, 11 August 2026

1. Scope of Work & Structural Overview

The Tripoli Emergency Fund (TEF) invites eligible contractors to submit sealed bids for the structural shoring, consolidation, and rehabilitation works for a residential building (Basement + Ground Floor + 6 Upper Floors) located on Plot No. 2054, Basatin Trablous.

The scope of this tender is strictly limited to the Basement Floor (≈ 303 m2) and Ground Floor (≈160 m2), covering the following primary structural and civil components:

- Column Jacketing: Concrete jacketing (10 cm thickness, f'c ≥ 35 MPa) for 8 columns, including surface roughening (5-10 mm amplitude), epoxy-anchored dowels/stirrups, 3x3 cm chamfered edges, and fair-faced finishes.

- Beam & Slab Jacketing: Structural repairs and section enlargement of drop/embedded beams and solid slabs (75 Lm) using structural repair mortar and bonding agents.

- Ribbed Slab Reconstruction: Complete demolition of existing failed ribbed slabs (250 m2) using saw-cutting and low-vibration equipment, followed by full reconstruction (rebar, top mesh T6@25cm, hollow block fill, and concrete pour).

- Surface Rehabilitation: Concrete patch repairs, crack injection, surface cleaning, and protective/waterproof coatings for Ground Floor areas (140 m2) and damaged upper surfaces/balconies (52 m2).

- Propping & Shoring: Temporary support systems utilizing double rows of adjustable steel props (1.2-1.5m spacing) extended one bay beyond repair zones, retained until concrete reaches ≥ 28 MPa or 90% design strength.

- Site Drainage & Civil Works: Installation of a new 60 LM UPVC drainage network ≥ 10 cm pipe diameter) with at least 4 concrete inspection chambers (40x40x40 cm) connected to municipal roadside manholes.

2. Key Details & Timeline

- Document Availability: Tuesday, 11 August 2026 (as of 3:00 PM) – Monday, 17 August 2026 (9:00 AM) via link/email.

- Mandatory Site Inspection: Monday, 17 August 2026 at 1:00 PM (Plot No. 2054, Basatin Trablous).

- Clarification Deadline: Wednesday, 19 August 2026 by 2:00 PM (GMT+3) via procurement@oea-tripoli.org.lb

- Submission Deadline: Friday, 21 August 2026 by 12:00 PM (GMT+3).

- Submission Location: OEA Premises, Procurement Unit (1st Floor) using a Three-Envelope System (Hard Copies).

3. Eligibility & Evaluation Criteria

A. Administrative, Legal & General Requirements (Pass/Fail)

- Mandatory Attendance: Site visit sign-off at Plot 2054.

- Professional Registration: Active membership in the Order of Engineers and Architects (Beirut/Tripoli) or the Lebanese Contractors Syndicate.

- Legal Compliance: Commercial Register & Circular, Authorized Signatories' IDs, Notarized Power of Attorney (if applicable), and Clean Judicial Record (سجل عدلي).

- Tax & Fiscal Clearances: Ministry of Finance Tax & VAT Certificates, UBO Declaration, NSSF Clearance (براءة ذمة), Commercial Court Non-Bankruptcy Certificate, and signed Integrity Annex.

- Bid Security: USD 1,000 bank guarantee (from an approved local bank or BDL), or OEA cash deposit receipt.

- General Site Provisions: Allowance for site notice board (2000x2500 mm reflective aluminum), full insurance coverage, comprehensive Health & Safety PPE/barriers/first aid, and final digital "As-Built" drawings.

B. Technical & Material Compliance

- Standards Adherence: Compliance with ACI 318-19, ACI 301, ACI 302, ACI 546, ASTM C31, C39, C143, C494, and E488.

- Approved Epoxy Anchoring: Use of two-component high-strength structural adhesive systems (equivalent to Hilti HIT-RE 500 V4, Hilti HIT-HY 200, Sika AnchorFix-3001, or Fischer FIS EM Plus).

- Quality Control & Testing: Mandatory pull-out proof testing for a minimum of 5% of installed epoxy dowels/anchors (proof load ≥ 1.25 x design load), concrete compressive strength tests, and cover meter verifications.

C. Financial & Experience Requirements

- General Experience: Minimum 7–10 years in structural concrete repair, shoring, or building rehabilitation.

- Specific Experience: Successful completion of at least two (2) structural rehabilitation, jacketing, or concrete repair projects within the last 5 years.

- Financial Audits: Audited financial statements for the past 3 fiscal years (2023, 2024, and 2025).

- Key Personnel:

  - Project Manager/Lead Structural Engineer: Civil Engineer with 10+ years of structural repair and concrete jacketing experience.

  - Site Engineer / Foreman: 5+ years of direct field experience in concrete patch repairs, structural shoring, and epoxy dowel installation.

Note: The OEA on behalf of the Tripoli Emergency Fund reserves the right to accept or reject any or all bids without providing justification.